| Laboratory Hazard Assessment Tool |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Activity Performed |  | Chemical Hazards |  |  |  |
| Yes | No | Activity in Lab | Potential Hazard | Active Researcher PPE | Additional PPE or Comments |
|  |  | Working with small volumes (<100mL) of corrosive liquids or solids (acids, caustics, etc) with little probability of a splash | Eye or skin damage. Low hazard for splash hazard | -Safety glasses -Chemical resistant gloves -Lab coat |  |
|  |  | Working with large volumes of corrosive liquids or solids (acids, caustics, etc) with the potential for a splash | Increased potential for eye and skin damage | -Safety goggles -Chemical-resistant gloves -Lab coat -chemical-resistant apron |  |
|  |  | Working with small volumes of acutely toxic liquid | Increased potential for eye and skin damage | -Safety goggles -Chemical-resistant gloves -Lab coat -chemical-resistant apron |  |
|  |  | Working with toxic or hazardous chemicals (solid, liquid or gas) including to and not limited to materials listed as a Health Hazard under GHS (GHS H301, H302, H311, H312, H331, H332) | Spills, splashes, ingestion, inhalation, absorption. Chemicals pose a high level of immediate health risk | -Safety glasses (Safety goggles for large amounts) -Chemical resistant gloves -Lab coat |  |
|  |  | Working with an apparatus with contents under pressure or vacuum | Eye or Skin damage | - safety glasses - chemical resistant gloves - Face Shield (high risk) activities) - Lab Coat |  |
| Laboratory Hazard Assessment Tool |  |  |  |  |  |
| Activity Performed |  | Chemical Hazards |  |  |  |
| Yes | No | Activity in Lab | Potential Hazard | Active Researcher PPE | Additional PPE or Comments |
|  |  | Working with flammable solvents: when there is a risk of ignition or areas where flammable vapors or gas may be present and reasonable ignition sources are present | Fire. Skin or eye damage, potential poisoning through skin contact | -Safety glasses -Chemical resistant gloves - FR glove liners -Flame-Resistant Lab coat |  |
|  |  | Working with pyrophoric materials outside of a glovebox | Fire. Skin or eye damage. | -Safety glasses -Chemical resistant gloves - FR glove liners -Flame-Resistant Lab coat -Face Shield |  |
|  |  | Working with air or water reactive chemicals outside of a glove box | Increased potential for eye and skin damage, flying debris, detonation. | -Safety glasses -Chemical resistant gloves - FR glove liners -Flame-Resistant Lab coat -Face Shield |  |
|  |  | Working with potentially explosive materials | Increased potential for eye and skin damage, flying debris, detonation. | -Safety glasses -Chemical resistant gloves - FR glove liners -Flame-Resistant Lab coat -Face Shield and/or blast shield |  |
| Laboratory Hazard Assessment Tool |  |  |  |  |  |
| Activity Performed |  | Chemical Hazards |  |  |  |
| Yes | No | Activity in Lab | Potential Hazard | Active Researcher PPE | Additional PPE or Comments |
|  |  | Major spill clean-up (any amount of acutely toxic chemical or other spills over 1 gallon) | Multiple Hazards | Call EHS for assistance |  |
|  |  | Working with known or suspected human carcinogens | Spills, splashes, ingestion, inhalation, absorption. High Hazard cancer-causing agents | -Safety glasses -Chemical resistant gloves -Lab coat |  |
|  |  | Working with known or suspected human carcinogens | Spills, splashes, ingestion, inhalation, absorption. Agents that affect reproductive capabilities cause mutation and adversely affect fetal development | -Safety glasses -Chemical resistant gloves -Lab coat |  |
|  |  | Working with engineered nanomaterials | Inhalation, exposure, dermal exposure, chemical exposure | -Safety glasses -Chemical resistant gloves - Lab coat |  |
|  |  | Minor Spill clean up | Skin, eye, or respiratory damage. | -Safety glasses -Chemical resistant gloves -Lab coat |  |

Laboratory Hazard Assessment Tool

| Activity Performed |  | Physical Hazards |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Yes | No | Activity in Lab | Potential Hazard | Active Researcher PPE | Additional PPE or Comments |
|  |  | Working with Cryogenic Liquids | Major skin, tissue or eye damage | -Safety glasses (goggles for large volumes) -Chemical resistant gloves -Lab coat -cryogenic protective gloves -Face shield |  |
|  |  | Removing freezer vials from Liquid Nitrogen | Increased potential for eye and skin damage | -Safety glasses -Chemical-resistant gloves -cryogenic protective gloves -Lab coat |  |
|  |  | Working with very cold equipment or dry ice | Increased potential for eye and skin damage | -Safety glasses (goggles for large volumes) -cryogenic protective gloves -Lab coat |  |
|  |  | Working with scalding liquids, hot equipment (autoclave, water, or oil bath) | Burns that could result in skin or eye damage | -Safety glasses (goggles for large volumes) -Thermal-protective gloves -Lab coat |  |
|  |  | Glassware washing |  | -Safety glasses -gloves -Lab coat |  |
| Laboratory Hazard Assessment Tool |  |  |  |  |  |
| Activity Performed |  | Physical Hazards |  |  |  |
| Yes | No | Activity in Lab | Potential Hazard | Active Researcher PPE | Additional PPE or Comments |
|  |  | Working with loud equipment, noises, sounds, alarms, etc | Potential hearing loss or ear damage | -Ear plugs or muffs as needed |  |
|  |  | Working with a centrifuge | Imbalance rotor can lead to broken vials, cuts, exposure | -Safety glasses -Disposable gloves -Lab coat |  |
|  |  | Working with a sonicator | Potential hearing loss or ear damage | -Safety glasses -Disposable gloves -Lab coat -Ear plugs or muffs as needed |  |
|  |  | Working with sharps | Cuts, exposure | -Safety glasses -Lab coat |  |

Laboratory Hazard Assessment Tool

| Activity Performed |  | Biological Hazards |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Yes | No | Activity in Lab | Potential Hazard | Active Researcher PPE | Additional PPE or Comments |
|  |  | Working with human blood, body fluids, cell lines (Primary or established), tissues, or bloodborne pathogens, or other potentially infectious material | Exposure to infectious material, sharps, injuries | -Safety glasses/goggles -Chemical resistant gloves -Barrier Lab coat |  |
|  |  | Work with animal and or human specimens preserved in a fixative (paraformaldehyde or formalin) | Increased potential for eye and skin damage | -Safety glasses/ goggles -Chemical resistant gloves -Lab coat |  |
|  |  | Working with microbial agents (bacteria, fungi, virus, parasites, yeast, recombinant DNA and or biological materials that may contain Risk group 1 agents or recombinant DNA (BSL-1) | Increased potential for eye and skin damage | -Safety glasses/goggles -Chemical-resistant gloves -Lab coat -chemical-resistant apron |  |
|  |  | Working with microbial agents (bacteria, fungi, virus, parasites, yeast, recombinant DNA and or biological materials that may contain Risk group 2 agents or recombinant DNA (BSL-2) | Exposure to infectious material, particularly through broken skin, mucous membranes, or sharps injuries. | -Safety glasses/goggles -Double layer of Chemical resistant gloves -Lab coat |  |
|  |  | Working with live animals alone or in conjunction with Risk group 1 | Animal bites, allergies, eye irritation, sharps injury. Exposure to infectious materials to individuals who may have personal health issues that make them more susceptible to infection | -Safety glasses/goggles -Disposable gloves -Lab coat |  |

Laboratory Hazard Assessment Tool

| Activity Performed |  | Radiological Hazards |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Yes | No | Activity in Lab | Potential Hazard | Active Researcher PPE | Additional PPE or Comments |
|  |  | Working with solid radioactive material or solid radioactive waste | Cell damage, potential spread of radioactive contamination | -Safety glasses -appropriate gloves for radioactive work -Lab coat |  |
|  |  | Working with liquid radioactive material or liquid radioactive waste | Cell damage, potential spread of radioactive contamination along with hazards produced by the specific chemical | -Safety glasses -Chemical-resistant gloves that are compatible with rad work -Lab coat |  |

Laboratory Hazard Assessment Tool

| Activity Performed |  | Non-Ionizing Radiation Hazards |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Yes | No | Activity in Lab | Potential Hazard | Active Researcher PPE | Additional PPE or Comments |
|  |  | Working with ultraviolet radiation | Corneal damage, Conjunctivitis, skin burns | -UV face shield or goggles -opaque gloves if potential hand exposure -Lab coat - other PPE: specify: |  |
|  |  | Working with infrared emitting equipment (glass-blowing) | Cataracts, burns to cornea, skin burns | -Appropriate filter safety glasses -FR Lab coat |  |

Laboratory Hazard Assessment Tool

| Activity Performed |  | Laser Hazards |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Yes | No | Activity in Lab | Potential Hazard | Active Researcher PPE | Additional PPE or Comments |
|  |  | Open Beam- Performing alignment, troubleshooting or maintenance that requires working with an open beam and/or defeating the interlock (s) on any Class 3 or Class 4 laser system. | Eye damage | - Appropriate laser safety goggles/glasses with optical density based on individual beam parameters |  |
|  |  | Open beam- viewing a Class 3R laser beam with magnifying optics | Eye damage | - Appropriate laser safety goggles/glasses with optical density based on individual beam parameters |  |
|  |  | Open beam- Working with Class 3B laser open beam system with potential for producing direct or specular reflections | Eye damage | - Appropriate laser safety goggles/glasses with optical density based on individual beam parameters |  |
|  |  | Open beam- Working with Class 3B laser open beam system with potential for producing direct or specular reflections | Eye Damage, skin damage | - Appropriate laser safety goggles/glasses with optical density based on individual beam parameters Appropriate skin protection |  |
|  |  | Non-beam- Handling of dye laser materials such as powdered dyes, chemicals, and solvents | Cancer, explosion, fire | - Safety glasses -Chemical-resistant gloves -Lab coat |  |
| Laboratory Hazard Assessment Tool |  |  |  |  |  |
| Activity Performed |  | Laser Hazards |  |  |  |
| Yes | No | Activity in Lab | Potential Hazard | Active Researcher PPE | Additional PPE or Comments |
|  |  | Non-beam- Maintaining and repairing power sources for large class 3B and class 4 laser systems | Electrocution, explosion, fire | -Electrical isolation mat -Flame resistant NEC 70E APC rated coveralls or lab coat |  |

Notes:  Please read any SDS related to the materials that you will be using.

The PPE in this assessment is the minimum PPE required for a certain task.  You may add more PPE as the lab wishes (including sleeve protectors, aprons, ___________

Regarding biological hazards, please follow any additional requirements as request by the IBC and IUAUC and note them in this Hazard assessment

Regarding laser hazards, please follow any requirements as requested by the Laser safety committee and note them in this Hazard assessment.

Laboratory Hazard Assessment Tool

| Activity Performed |  | Lab Specific Activities If your lab conducts any additional activities that is not listed above, identify the hazards and appropriate PPE, then add them to the table below. Add pages as needed |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Yes | No | Activity in Lab | Potential Hazard | Active Researcher PPE | Additional PPE or Comments |

Laboratory Hazard Assessment Tool

Please certify that you have conducted this hazard assessment by filling out and signing this page.  Maintain a copy of the signed hazard assessment with your CHP.

Name and Title of person conducting assessment

Principal Investigator’s name (Print name)_________________________________________

Building and Lab rooms:________________________________________________________

Signature of PI:_______________________________________________________________

Date assessment completed: ____________________________________________________

Laboratory Hazard Assessment Tool

The following employees have reviewed this Laboratory Hazard Assessment Tool specific to this laboratory and have received the following training:

- When PPE is necessary

- What PPE is required

- How to properly don, doff, adjust and wear PPE

- The limitations of PPE

- The proper care, maintenance, useful life and PPE disposal

- General PPE best practices (not wearing PPE outside, in the bathroom, eating areas, etc)

| Date | Trainer | Name of Trainee | Worker Signature |
| --- | --- | --- | --- |

Laboratory Hazard Assessment Tool

The following employees have reviewed this Laboratory Hazard Assessment Tool specific to this laboratory and have received the following training:

- When PPE is necessary

- What PPE is required

- How to properly don, doff, adjust and wear PPE

- The limitations of PPE

- The proper care, maintenance, useful life and PPE disposal

- General PPE best practices (not wearing PPE outside, in the bathroom, eating areas, etc)

| Date | Trainer | Name of Trainee | Worker Signature |
| --- | --- | --- | --- |